Straw Tube Tracker

Requirements:

- 150 um position resolution on hit straws (giving a 1mr angular resolution with 2 chambers).

- At least 95% efficiency (giving better than 99.999% efficiency for 3 out of 5).

- Time resolution not critical.

- Position repeatability of the chamber to within 10-20um (Ron – what do you think?) The table should be repeatable to 10 um. Are you referring to wire to wire spacing or something else?

Steps taken to ensure this

Resolution:

- Test individual straws with a radioactive source.

- Test multiple planes with source.

- Test chambers at PSI with GEM tracking.

Efficiency:

- Test straws/planes with cosmics.

- Online testing at PSI.

Repeatability:

- Tests at PSI with GEMs/Frames.

SciFi Detector

Requirements:

- Timing resolution better then 1ns per fiber (becomes better than ~700ps when demanding 2 planes).

- Position resolution – Fiber size 2mm.

- Efficiency better then 95%/plane (better than 99% when demanding 2/3 planes).

Steps taken to ensure this

Resolution and timing:

- Cosmics tests at TAU.

- Beam tests at PSI.

Efficiency:

- Cosmics tests at TAU.

- Online testing at PSI.

Detector Frame Requirements

Requirements:

- Rotary stage position reproducible to about 0.5 mil or 10 um, corresponding to angular repeatability of < 0.1 mr.

- Translation stage movement distance known to 1 mm, with angle changes less than 0.1 mr horizontal x 1 mr vertical.

- Supports detectors.

Steps taken to ensure this

- Careful mechanical construction

- Survey

- laser pointing

Beam Cerenkov Detectors

Requirements:

- Timing resolution better then 0.1 ns.

- Efficiency about 99% or better for muons.

Steps taken to ensure this

Timing resolution:

- Cosmic and beam tests at PSI, with respect to fact scintillator.

- Vary angle of Cerenkov wrt beam.

Efficiency:

- Beam testing at PSI, with tracking or scintillator stack.